# Laboratory Work 2

## Course

Microcontroller Systems

## Title

Clock Configuration, SysTick 10 kHz and Output Frequency Measurement

## Objective

To extend Lab 1 firmware with system clock setup logic, periodic interrupt generation, and measurable output signal toggling.

## Variant

Variant: 5 Required frequency by task table: 80 MHz.

## Task Requirements

- Configure flash latency.

- Configure PLL source and PLL factors.

- Enable PLL and switch main clock source.

- Start SysTick timer at 10 kHz.

- Toggle GPIO pin inside SysTick handler.

- Measure output signal frequency in simulation tools.

## Schematic / Simulation Setup

Runtime demonstration was assembled and executed in Wokwi.

- Scheme view:

Lab 2 Wokwi scheme

- Logic analyzer running view:

Lab 2 logic analyzer

- Frequency demonstration with active runtime state:

Lab 2 frequency value

## Source Code

Primary STM32 implementation file: - Lab2_main.c

Wokwi runtime analogue sketch file: - Wokwi_Lab2_sketch.ino

## Calculations (Variant 5)

Required target frequency from variant table: - f_SYSCLK = 80 MHz

PLL setup used in STM32 code: - HSI = 16 MHz - PLLM = 16 - PLLN = 320 - PLLP = 4

Check: 1. f_VCO_IN = 16 / 16 = 1 MHz 2. f_VCO_OUT = 1 * 320 = 320 MHz 3. f_SYSCLK = 320 / 4 = 80 MHz

SysTick setup: - required interrupt frequency: 10 kHz - reload value: LOAD = 80,000,000 / 10,000 - 1 = 7999

Output signal behavior: - output pin toggles on each tick, - expected square-wave fundamental frequency is approximately: - f_OUT ≈ 10 kHz / 2 = 5 kHz

## Verification Notes

- LED/button behavior from Lab 1 remains operational.

- Output pin signal is generated continuously.

- Logic analyzer confirms periodic toggling.

## Code Listing (main.c)

#include "stm32f4xx.h"
#include <stdint.h>

#ifndef LAB_VARIANT
#define LAB_VARIANT 5u
#endif

#define BTN_UP_PIN      (0u)   // PA0
#define BTN_DOWN_PIN    (1u)   // PA1
#define LED_MASK_8BIT   (0xFFu) // PC0..PC7
#define FREQ_OUT_PIN    (0u)   // PB0 for oscilloscope

static volatile uint8_t g_counter = 0u;

static uint32_t target_freq_mhz_from_variant(uint32_t variant)
{
    if (variant < 1u) {
        return 84u;
    }
    if (variant > 41u) {
        return 44u;
    }
    return 85u - variant;
}

static void delay_cycles(volatile uint32_t n)
{
    while (n--) {
        __NOP();
    }
}

static uint8_t button_pressed(uint32_t pin)
{
    return ((GPIOA->IDR & (1u << pin)) == 0u) ? 1u : 0u;
}

static void leds_write(uint8_t value)
{
    uint32_t odr = GPIOC->ODR;
    odr &= ~LED_MASK_8BIT;
    odr |= value;
    GPIOC->ODR = odr;
}

static void gpio_init(void)
{
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN | RCC_AHB1ENR_GPIOBEN | RCC_AHB1ENR_GPIOCEN;
    (void)RCC->AHB1ENR;

    // PC0..PC7 as LEDs.
    GPIOC->MODER &= ~(0xFFFFu);
    GPIOC->MODER |=  (0x5555u);
    GPIOC->OTYPER &= ~(LED_MASK_8BIT);
    GPIOC->PUPDR  &= ~(0xFFFFu);

    // PA0/PA1 as button inputs with pull-up.
    GPIOA->MODER &= ~((3u << (BTN_UP_PIN * 2u)) | (3u << (BTN_DOWN_PIN * 2u)));
    GPIOA->PUPDR &= ~((3u << (BTN_UP_PIN * 2u)) | (3u << (BTN_DOWN_PIN * 2u)));
    GPIOA->PUPDR |=  ((1u << (BTN_UP_PIN * 2u)) | (1u << (BTN_DOWN_PIN * 2u)));

    // PB0 as output for frequency observation.
    GPIOB->MODER &= ~(3u << (FREQ_OUT_PIN * 2u));
    GPIOB->MODER |=  (1u << (FREQ_OUT_PIN * 2u));
    GPIOB->OTYPER &= ~(1u << FREQ_OUT_PIN);
    GPIOB->PUPDR  &= ~(3u << (FREQ_OUT_PIN * 2u));
}

static void set_flash_latency(uint32_t sysclk_mhz)
{
    uint32_t latency;

    if (sysclk_mhz <= 30u) {
        latency = FLASH_ACR_LATENCY_0WS;
    } else if (sysclk_mhz <= 60u) {
        latency = FLASH_ACR_LATENCY_1WS;
    } else {
        latency = FLASH_ACR_LATENCY_2WS;
    }

    FLASH->ACR = FLASH_ACR_ICEN | FLASH_ACR_DCEN | FLASH_ACR_PRFTEN | latency;
    while ((FLASH->ACR & FLASH_ACR_LATENCY) != latency) {
    }
}

static void clock_init_pll_hsi(uint32_t target_mhz)
{
    uint32_t pllm = 16u;
    uint32_t pllp = 4u;
    uint32_t plln;

    // Keep HSI on.
    RCC->CR |= RCC_CR_HSION;
    while ((RCC->CR & RCC_CR_HSIRDY) == 0u) {
    }

    // Temporarily switch SYSCLK to HSI before touching PLL.
    RCC->CFGR &= ~RCC_CFGR_SW;
    while ((RCC->CFGR & RCC_CFGR_SWS) != RCC_CFGR_SWS_HSI) {
    }

    RCC->CR &= ~RCC_CR_PLLON;
    while ((RCC->CR & RCC_CR_PLLRDY) != 0u) {
    }

    // Choose PLLP so PLLN stays in valid range.
    if ((target_mhz * 4u) >= 192u) {
        pllp = 4u;
    } else {
        pllp = 6u;
    }
    plln = target_mhz * pllp;

    set_flash_latency(target_mhz);

    // AHB = SYSCLK; APB1 <= 42 MHz; APB2 <= 84 MHz.
    RCC->CFGR &= ~(RCC_CFGR_HPRE | RCC_CFGR_PPRE1 | RCC_CFGR_PPRE2);
    RCC->CFGR |= RCC_CFGR_HPRE_DIV1 | RCC_CFGR_PPRE1_DIV2 | RCC_CFGR_PPRE2_DIV1;

    RCC->PLLCFGR = (pllm & RCC_PLLCFGR_PLLM)
                 | ((plln << RCC_PLLCFGR_PLLN_Pos) & RCC_PLLCFGR_PLLN)
                 | ((((pllp / 2u) - 1u) << RCC_PLLCFGR_PLLP_Pos) & RCC_PLLCFGR_PLLP)
                 | RCC_PLLCFGR_PLLSRC_HSI;

    RCC->CR |= RCC_CR_PLLON;
    while ((RCC->CR & RCC_CR_PLLRDY) == 0u) {
    }

    RCC->CFGR &= ~RCC_CFGR_SW;
    RCC->CFGR |= RCC_CFGR_SW_PLL;
    while ((RCC->CFGR & RCC_CFGR_SWS) != RCC_CFGR_SWS_PLL) {
    }
}

static void systick_init_10khz(uint32_t core_clk_hz)
{
    SysTick->LOAD = (core_clk_hz / 10000u) - 1u;
    SysTick->VAL = 0u;
    SysTick->CTRL = SysTick_CTRL_CLKSOURCE_Msk
                  | SysTick_CTRL_TICKINT_Msk
                  | SysTick_CTRL_ENABLE_Msk;
}

void SysTick_Handler(void)
{
    // Toggle PB0 at each SysTick interrupt.
    GPIOB->ODR ^= (1u << FREQ_OUT_PIN);
}

int main(void)
{
    const uint32_t target_mhz = target_freq_mhz_from_variant(LAB_VARIANT);
    uint8_t up_prev = 0u;
    uint8_t down_prev = 0u;

    clock_init_pll_hsi(target_mhz);
    gpio_init();
    leds_write(g_counter);

    // Required by task: SysTick at 10 kHz.
    systick_init_10khz(target_mhz * 1000000u);

    for (;;) {
        uint8_t up_now = button_pressed(BTN_UP_PIN);
        uint8_t down_now = button_pressed(BTN_DOWN_PIN);

        if ((up_now != 0u) && (up_prev == 0u)) {
            g_counter++;
            leds_write(g_counter);
            delay_cycles(120000u);
        }

        if ((down_now != 0u) && (down_prev == 0u)) {
            g_counter--;
            leds_write(g_counter);
            delay_cycles(120000u);
        }

        up_prev = up_now;
        down_prev = down_now;
    }
}

## Conclusion

The second laboratory task objectives were achieved: clock-configuration logic was implemented, 10 kHz periodic timing was established, and output frequency behavior was demonstrated and captured.