# Laboratory Work 3

## Course

Microcontroller Systems

## Title

USART Communication with TX/RX Handling and LED Control

## Objective

To extend the previous project with USART packet transmission on button press and byte reception for LED control according to variant rules.

## Variant

Variant: 5

RX command mapping: - 4 -> LED OFF - 5 -> LED ON

## Task Requirements and Implementation

- Send function for byte packet: implemented as sendPacketIRQStyle().

- Transmit flow with TXE-like behavior: byte-by-byte transmission is performed in software loop (functional simulation analogue).

- Button-triggered sending: pushbutton on digital pin 2 launches packet transmission.

- RX processing: received byte is checked and LED on digital pin 4 is switched ON/OFF for variant values.

## Circuit / Simulation Setup

Functional simulation setup was implemented in Wokwi: - Controller board: Arduino Uno (used as simulation analogue due stable UART terminal support in Wokwi). - Pushbutton: D2 to GND (active low with internal pull-up). - LED line: D4 -> R(330 Ohm) -> LED -> GND. - UART terminal: built-in Serial Monitor (9600 baud).

Schematic screenshot:

Lab 3 scheme

## Runtime Verification

TX packet after button press:

Lab 3 UART TX

RX command processing and LED control:

Lab 3 UART RX and LED

Observed behavior: 1. On startup, Serial terminal prints initialization text. 2. Button press sends packet USART IRQ packet. 3. Entering 5 in terminal turns LED ON. 4. Entering 4 in terminal turns LED OFF.

## Calculations

USART baud in simulation: - Configured baud rate: 9600 bps. - Terminal baud rate: 9600 bps. - Baud mismatch in this setup: 0%.

Variant logic check: - Task table for variant 5: 4-off, 5-on. - Implemented mapping matches the task.

## Code Listing (Wokwi_Lab3_sketch.ino)

const int BTN_SEND = 2;
const int LED_CTRL = 4;

const char CMD_OFF = '4';
const char CMD_ON  = '5';

bool btnPrev = false;

void setup() {
  pinMode(BTN_SEND, INPUT_PULLUP);
  pinMode(LED_CTRL, OUTPUT);
  digitalWrite(LED_CTRL, LOW);

  Serial.begin(9600);
  delay(500);
  Serial.println("Lab3 start");
  Serial.println("Press button to send packet");
  Serial.println("Send '5' to turn LED ON, '4' to turn LED OFF");
}

void sendPacketIRQStyle(const char *s) {
  while (*s) {
    Serial.write(*s++);
    delay(1);
  }
}

void loop() {
  bool btnNow = (digitalRead(BTN_SEND) == LOW);

  if (btnNow && !btnPrev) {
    sendPacketIRQStyle("USART IRQ packet\\r\\n");
  }
  btnPrev = btnNow;

  while (Serial.available() > 0) {
    char rx = (char)Serial.read();

    if (rx == CMD_ON) {
      digitalWrite(LED_CTRL, HIGH);
      Serial.println("LED ON");
    } else if (rx == CMD_OFF) {
      digitalWrite(LED_CTRL, LOW);
      Serial.println("LED OFF");
    }
  }
}

Additional microcontroller implementation file (STM32 source used in project structure): - Lab3_main.c

## Conclusion

Laboratory work 3 was completed. Packet transmission on button press and receive-based LED control were implemented and verified. Variant 5 conditions (4/5) are fully satisfied.